A DISTRIBUTED ALGORITHM OF APPLIANCE SCHEDULING FOR

HOME ENERGY MANAGEMENT SYSTEM

ABSTRACT

Demand side management encourages the users in a smart grid to shift their electricity consumption in response to varying electricity prices. In this paper, we propose a distributed framework for the demand response based on cost minimization. Each user in the system will find an optimal start time and operating mode for the appliances in response to the varying electricity prices. We model the cost function for each user and the constraints for the appliances. We then propose an approximate greedy iterative algorithm that can be employed by each user to schedule appliances. In the proposed algorithm, each user requires only the knowledge of the price of the electricity, which depends on the aggregated load of other users, instead of the load profiles of individual users. In order for the users to coordinate with each other, we introduce a penalty term in the cost function, which penalizes large changes in the scheduling between successive iterations. Numerical simulations show that our optimization method will result in lower cost for the consumers, lower generation costs for the utility companies, lower peak load, and lower load fluctuations.

INTRODUCTION

In this paper, we propose a distributed energy scheduling algorithm as a demand response for the smart grid. We use day-ahead pricing scheme, where the price of the electricity for the day is determined on the previous day. We then find optimal operating times for the electric appliances and their corresponding energy consumptions by minimizing the overall cost of operation. Our approach is different from the related work in four main aspects: i) we jointly optimize both the start time and the energy consumption for each appliance of the user; ii) we bill all the users based on their time-dependent use of electricity; iii) we enforce realistic constraints on the operation of the appliances by categorizing them into two different classes; iv) we let the energy consumption vary in a discrete manner, which is more realistic. Further, our algorithm is fully distributed, where the only information available to the users is the prices for different

time-periods. Using this price, each user will find his energy consumption schedule. Since we allow the energy consumption to vary in a discrete fashion, the corresponding optimization problem will be NP-hard. Therefore, we employ a greedy iterative algorithm to find the sub-optimal energy consumption schedule of each user. In each iteration, all the users will communicate their energy consumption schedule to the utility company. The utility company will then adjust the price depending on the overall system load and broadcast the price to all the users. The users will then update their energy consumption based on the new price. These iterations continue until convergence. We use numerical simulations to show that the proposed algorithm will result in lower cost for the consumers, higher profit for the utility companies, lower peak load, and lower load variance. The rest of this paper is organized as follows. In Section II we introduce the system model, where we describe the model for the users, model for the appliances, and the pricing strategy. We also formulate the problem of appliance scheduling as a centralized optimization problem. In Section III, we propose a distributed framework to solve the optimization problem, where each user employs a greedy approach. We provide numerical simulations in Section IV, and conclude the paper in Section V.

BLOCK DIAGRAM

TRANSFORMER

LOAD

RELAY’S

POWER SUPPLY

CIRCUIT

ZIGBEE

KEYPAD

16 X 2 LCD

Micro controller

TRANSMITTED BLOCK

PC

MAX 232

ZIGBEE

RECEIVER BLOCK

HARDWARE USED

- MICRO CONTROLLER

- 16X2 ,LCD

- ZIBEE

- KEYPAD

- CURRENT TRANSFORMER

- RELAY

- PERSONAL COMPUTER

SOFTWARE USED

- MPLAB,

- CCS C COMPILER

- TERMINAL WINDOW