GPS AND GSM BASED REAL-TIME HUMAN HEALTH MONITORING AND ALERT SYSTEM FOR CARDIAC PATIENTS

ABSTRACT:

The primary function of this system is to monitor the temperature and Heart Beat of the Patient and the Data collected by the sensors are sent to the Microcontroller. The Microcontroller transmits the data over the air. Here we are using the GSM modem in order to transmit the information.

From the transmitter the recordings are sent as an SMS to the care taker or the expert which have been given as the recipient. Not only we send the information through GSM module as SMS we also display the readings on LCD. And when the conditions go abnormal then we sense those values and then alarm the people around by blowing the alarm and also sending by sending as urgency SMS.

The functioning of Heart Beat device is based on the truth that the blood circulates for every heartbeat that can be sensed by using a circuit formed by the combination of a LDR and LED. Depending upon the rate of circulation of blood the heart beat rate per minute is calculated. This calculated value is communicated to remote person through a GSM modem interface to it.

Here we tacking the input parameter as a heart beat   this is sensed by the sensor circuit and then heart beat calculate by the real time clock. If the heart beat is greater than the predefined level the information send to the concerned person.

This project uses regulated 5V, 750mA power supply. 7805 three terminal voltage regulator is used for voltage regulation. Bridge type full wave rectifier is used to rectify the ac output of secondary of 230/18V step down transformer.

BLOCK DIAGRAM:

Temperature sensor

Controller

LCD

A

D

C

Heart beat sensor rate sensor

GSM MODEM

Vibration sensor

GPS MODEM

POWER SUPPLY:

Regulator

Filter

Rectifier

Step down transformer

Power supply

HARDWARE REQUIREMENTS:

- Micro Controller

- Power Supply

- GSM modem

- GPS  modem

- ADC

- Temperature sensor

- Heart beat sensor

- Vibration sensor

- Buzzer

- LCD display

SOFTWARE REQUIREMENTS:

- KEIL software

- RIDE software

ADVANTAGES:

  - Wireless patient monitoring system in hospitals

  - Continuously patient monitoring and take right action on right time.

  - Easily check out the patient condition on predefined mobile numbers.

  - Continuously sends sms to the doctor about patient condition.

  - Easily applicable.

APPLICATIONS:

  - Wireless patient monitoring system in hospitals

  - Remote areas

  - Biomedical applications